Network Layer

Logical Addressing

Communication at the network layer is host-to-host (computer-to-computer); a computer somewhere in the world needs to communicate with another computer somewhere else in the world. Usually, computers communicate through the Internet. The packet transmitted by the sending computer may pass through several LANs or WANs before reaching the destination computer. For this level of communication, we need a global addressing scheme; we called this logical addressing. Today, we use the term IP address to mean a logical address in the network layer of the TCP/IP protocol suite. The Internet addresses are 32 bits in length; this gives us a maximum of 232 addresses. These addresses are referred to as IPv4 (IP version 4) addresses or simply IP addresses. The need for more addresses motivated a new design of the IP layer called the new generation of IP or IPv6 (lP version 6). In this version, the Internet uses 128-bit addresses that give much greater flexibility in address allocation. These addresses are referred to as IPv6 (IP version 6) addresses. In this chapter, we first discuss IPv4 addresses, which are currently being used in the Internet. We then discuss the IPv6 addresses, which may become dominant in the future.

Address Space

A protocol such as IPv4 that defines addresses has an address space. An address space is the total number of addresses used by the protocol. If a protocol uses N bits to define an address, the address space is 2N because each bi tcan have two different values (0 or 1) and N bits can have 2N values. IPv4 uses 32-bit addresses, which means that the address space is 232 or 4,294,967,296 (more than 4 billion). This means that, theoretically, if there were no restrictions, more than 4 billion devices could be connected to the Internet. We will see shortly that the actual number is much less because of the restrictions imposed on the addresses.

IPv4 ADDRESSES

An IPv4 address is a 32-bit address that uniquely and universally defines the connection of a device (for example, a computer or a router) to the Internet. The address space of IPv4 is 232

There are two prevalent notations to show an IPv4 address: binary notation and dotted decimal notation.

Binary Notation: 		01110101 10010101 00011101 00000010

Dotted-Decimal Notation:	117.149.29.2

Classful Addressing

In classful addressing, the address space is divided into five classes: A, B, C, D, and E. Each class occupies some part of the address space.

If the address is given in binary notation, the first few bits can immediately tell us the class of the address. If the address is given in decimal-dotted notation, the first byte defines the class

|  | Binary notation (Starts with) | Dotted decimal (First byte) | Application |
| --- | --- | --- | --- |
| Class A | 0 | 0-127 | Unicast |
| Class B | 10 | 128-191 | Unicast |
| Class C | 110 | 192-223 | Unicast |
| Class D | 1110 | 224-239 | Multicat |
| Class E | 1111 | 240-255 | Reserved |

Netid and Hostid

In classful addressing, an IP address in class A, B, or C is divided into netid and hostid. These parts are of varying lengths, depending on the class of the address. The concept does not apply to classes D and E.

In class A, one byte defines the netid and three bytes define the hostid. In class B, two bytes define the netid and two bytes define the hostid. In class C, three bytes define the netid and one byte defines the hostid.

Mask

The length of the netid and hostid (in bits) is predetermined in classful addressing, we can also use a mask (also called the default mask), a 32-bit number made of contiguous 1s followed by contiguous 0s. The masks for classes A, B, and C are shown in the following Table. The concept does not apply to classes D and E.

| Class | Binary notation | Dotted decimal | CIDR |
| --- | --- | --- | --- |
| A | 11111111 00000000 00000000 0000000 | 255.0.0.0 | /8 |
| B | 11111111 11111111 00000000 0000000 | 255. 255.0.0 | /16 |
| C | 11111111 11111111 11111111 0000000 | 255. 255. 255.0 | /24 |

The mask can help us to find the netid and the hostid. For example, the mask for a class A address has eight 1s, which means the first 8 bits of any address in class A define the netid; the next 24 bits define the hostid.

The last column of above table shows the mask in the form /n where n can be 8, 16, or 24 in classful addressing. This notation is also called slash notation or Classless Interdomain Routing (CIDR) notation. The notation is used in classless addressing.

Subnetting

If an organization was granted a large block in class A or B, it could divide the addresses into several contiguous groups and assign each group to smaller networks called subnets

Supernetting

The size of a class C block with a maximum number of 256 addresses did not satisfy the needs of most organizations. In supernetting, an organization can combine several class C blocks to create a larger range of addresses.

Classless Addressing

In this scheme, there are no classes, but the addresses are still granted in blocks.

Address Blocks

In classless addressing, when an entity, small or large, needs to be connected to the Internet, it is granted a block (range) of addresses. The size of the block (the number of addresses) varies based on the nature and size of the entity. For example, a household may be given only two addresses; a large organization may be given thousands of addresses. An ISP, as the Internet service provider, may be given thousands or hundreds of thousands based on the number of customers it may serve.

Restriction To simplify the handling of addresses, the Internet authorities impose three restrictions on classless address blocks:

- The addresses in a block must be contiguous, one after another.

- The number of addresses in a block must be a power of 2 (I, 2, 4, 8, ... ).

- The first address must be evenly divisible by the number of addresses.

Mask

In which the n leftmost bits are 1s and the 32 - n rightmost bits are 0s. However, in classless addressing the mask for a block can take any value from 0 to 32. It is very convenient to give just the value of n preceded by a slash (CIDR notation).

Eg. 205.16.37.32/28 , in which 205.16.37.32 defines one of the addresses and the /28 defines the mask.

- The first address in the block can be found by setting the rightmost 32 - n bits to 0s

- The last address in the block can be found by setting the rightmost 32 - n bits to 1s

- The number of addresses in the block can be found by using the formula 232- n.

Eg, 205.16.37.39/28

First address in the block:

The binary representation of the given address is 11001101 00010000 00100101 001001111. If we set 32 - 28 rightmost bits to 0, we get 11001101 000100000100101 0010000 or 205.16.37.32.

Last address in the block:

The binary representation of the given address is 11001101 000100000010010100100111. If we set 32 - 28 rightmost bits to 1, we get 11001101 00010000 001001010010 1111 or 205.16.37.47.

Length of the block:

The value of n is 28, which means that number of addresses is 232- 28 or 16.

Network Addresses

The first address in a block is normally not assigned to any device; it is used as the network address that represents the organization to the rest of the world.

Hierarchy

- Two-Level Hierarchy: No Subnetting

An IP address can define only two levels of hierarchy when not subnetted. The n leftmost bits of the address x.y.z.t/n define the network (organization network); the 32 – n rightmost bits define the particular host (computer or router) to the network. The two common terms are prefix and suffix. The part of the address that defines the network is called the prefix; the part that defines the host is called the suffix. The prefix is common to all addresses in the network; the suffix changes from one device to another.

- Three-Levels of Hierarchy: Subnetting

In this the address has 3 parts, network prefix, subnet prefix and host address.

As an example, suppose an organization is given the block 17.12.40.0/26, which contains 64 addresses. The organization has three offices and needs to divide the addresses into three sub blocks of 32, 16, and 16 addresses. We can find the new masks by using the following arguments:

- Suppose the mask for the first subnet is n1, then 232- n1 must be 32, which means that n1 =27.

- Suppose the mask for the second subnet is n2, then 232- n2 must be 16, which means that n2 = 28.

- Suppose the mask for the third subnet is n3, then 232- n3 must be 16, which means that n3 =28.

This means that we have the masks 27, 28, 28 with the organization mask being 26.

IPv6 ADDRESSES

An IPv6 address consists of 16 bytes (octets); it is 128 bits long. IPv6 has a much larger address space; 2128 addresses are available.

An IPv6 address is represented as eight groups of four hexadecimal digits, each group representing 16 bits .The groups are separated by colons (:). An example of an IPv6 address is:

2001:0db8:85a3:0000:0000:8a2e:0370:7334

The standards provide flexibility in the representation of IPv6 addresses. The full representation of eight four-digit groups may be simplified by several techniques, eliminating parts of the representation. In general, representations are shortened as much as possible. The hexadecimal digits are always compared in case-insensitive manner, but IETF recommendations suggest the use of only lower case letters. For example, 2001:db8::1 is preferred over 2001:DB8::1.

Leading zeros in each 16-bit field are suppressed, but each group must retain at least one digit in the case of the all-zero group. For example, 2001:0db8::0001:0000 is rendered as 2001:db8::1:0. The all-zero field that is explicitly presented is rendered as 0.

The longest sequence of consecutive all-zero fields is replaced with two colons ("::"). If the address contains multiple runs of all-zero fields, then it is the leftmost that is compressed to prevent ambiguities. For example, 2001:db8:0:0:1:0:0:1 is rendered as 2001:db8::1:0:0:1 rather than as 2001:db8:0:0:1::1..

IPv6 addresses are classified by the primary addressing and routing methodologies common in networking. Types of IPV6 addresses are

1.Unicast Addresses

A unicast address defines a single computer. The packet sent to a unicast address must be delivered to that specific computer. IPv6 defines two types of unicast addresses: geographically based and provider-based

. The provider-based address is generally used by a normal host as a unicast address

| 3 | 5 | Provider idenifier | Subscriber Identifier | Subnet Identifier | Node Identifier |
| --- | --- | --- | --- | --- | --- |

.

Provider prefix

Subscriber prefix

Subnet prefix

Fields for the provider-based address are as follows:

- Type identifier. This 3-bit field defines the address as a provider-based address.

- Registry identifier. This 5-bit field indicates the agency that has registered the address. Currently three registry centers have been defined. INTERNIC (code 11000) is the center for North America; RIPNIC (code 01000) is the center for European registration; and APNIC (code 10100) is for Asian and Pacific countries.

- Provider identifier. This variable-length field identifies the provider for Internet access (such as an ISP). A 16-bit length is recommended for this field.

- Subscriber identifier. When an organization subscribes to the Internet through a provider, it is assigned a subscriber identification. A 24-bit length is recommended for this field.

- Subnet identifier. Each subscriber can have many different subnetworks, and each subnetwork can have an identifier. The subnet identifier defines a specific subnetwork under the territory of the subscriber. A 32-bit length is recommended for this field.

- Node identifier. The last field defines the identity of the node connected to a subnet. A length of 48 bits is recommended for this field to make it compatible with the 48-bit link (physical) address used by Ethernet. In the future, this link address will probably be the same as the node physical address.

2.Multicast Addresses

Multicast addresses are used to define a group of hosts instead of just one. A packet sent to a multicast address must be delivered to each member of the group.

The second field is a flag that defines the group address as either permanent or transient. A permanent group address is defined by the Internet authorities and can be accessed at all times. A transient group address, on the other hand, is used only temporarily. Systems engaged in a teleconference, for example, can use a transient group address. The third field defines the scope of the group address.

3.Reserved Addresses

Another category in the address space is the reserved address. These addresses start with eight 0s (type prefix is 00000000). A few subcategories are defined in this category as follows

| 00000000 | 120 bits (all 0s) | unspecified |
| --- | --- | --- |

| 00000000 | 0000…..00001(120 bits ) | loopback |
| --- | --- | --- |

| 00000000 | 88 bits (all 0s) | IPv4 address | compatible |
| --- | --- | --- | --- |

| 00000000 | 72 bits (all 0s) | 16 bits(all 1s) | IPv4 address | Mapped |
| --- | --- | --- | --- | --- |

An unspecified address is used when a host does not know its own address and sends an inquiry to find its address. A loopback address is used by a host to test itself without going into the network. A compatible address is used during the transition from IPv4 to IPv6. It is used when a computer using IPv6 wants to send a message to another computer using IPv6, but the message needs to pass through a part of the network that still operates in IPv4. A mapped address is also used during transition. However, it is used when a computer that has migrated to IPv6 wants to send a packet to a computer still using IPv4.

4. Local Addresses

These addresses are used when an organization wants to use IPv6 protocol without being

connected to the global Internet. In other words, they provide addressing for private networks. Nobody outside the organization can send a message to the nodes using these addresses. Two types of addresses are defined for this purpose,

| 1111101010 | 70 bits (all 0s) | Node address(48 bits) | Link local |
| --- | --- | --- | --- |

| 1111101011 | 38 bits (all 0s) | 32 bits(subnet) | Node address | Site local |
| --- | --- | --- | --- | --- |

A link local address is used in an isolated subnet; a site local address is used in an isolated site with several subnets.

IPv4 Protocol

The Internet Protocol version 4 (IPv4) is the delivery mechanism used by the TCP/IP protocols. IPv4 is an unreliable and connectionless datagram protocol-a best-effort delivery service. The term best-effort means that IPv4 provides no error control or flow control (except for error detection on the header). IPv4 assumes the unreliability of the underlying layers and does its best to get a transmission through to its destination, but with no guarantees. If reliability is important, IPv4 must be paired with a reliable protocol such as TCP.

IPv4 is also a connectionless protocol for a packet-switching network that uses the datagram approach. This means that each datagram is handled independently, and each datagram can follow a different route to the destination. This implies that datagrams sent by the same source to the same destination could arrive out of order. Also, some could be lost or corrupted during transmission. Again, IPv4 relies on a higher-level protocol to take care of all these problems.

Datagram

Packets in the IPv4 layer are called datagrams. A datagram is a variable-length packet consisting of two parts: header and data. The header is 20 to 60 bytes in length and contains information essential to routing and delivery. The important fields in header section are

- Version (VER). This 4-bit field defines the version of the IPv4 protocol. This field tells the IPv4 software running in the processing machine that the datagram has the format of version 4. All fields must be interpreted as specified in the fourth version of the protocol. If the machine is using some other version of IPv4, the datagram is discarded rather than interpreted incorrectly.

- Header length (HLEN). This 4-bit field defines the total length of the datagram header in 4-byte words. This field is needed because the length of the header is variable (between 20 and 60 bytes). When there are no options, the header length is 20 bytes, and the value of this field is 5 (5 x 4 = 20). When the option field is at its maximum size, the value of this field is 15 (15 x 4 = 60).

- Services. This 8-bit field previously called service type, is now called differentiated services.

  - Service Type

In this interpretation, the first 3 bits are called precedence bits. Precedence is a 3-bit subfield ranging from 0 (000 in binary) to 7 (111 in binary). The precedence defines the priority of the datagram in issues such as congestion. The next 4 bits are called type of service (TOS) bits, and the last bit is not used. TOS bits is a 4-bit subfield with each bit having a special meaning. Although a bit can be either 0 or 1, one and only one of the bits can have the value of 1 in each datagram.

Application programs can request a specific type of service

  - Differentiated Services

In this interpretation, the first 6 bits make up the codepoint subfield, and the last 2 bits are not used

- Total length. The total length field defines the total length of the datagram including the header

- Identification. This field is used in fragmentation. This 16-bit field identifies a datagram originating from the source host. defined below

- Flags. This 3 bit field is used in fragmentation defined below

- Fragmentation offset. This field is used in fragmentation. This 13-bit field shows the relative position of this fragment with respect to the whole datagram. defined below

- Time to live. A datagram has a limited lifetime in its travel through an internet. This field is used mostly to control the maximum number of hops (routers) visited by the datagram. When a source host sends the datagram, it stores a number in this field. Each router that processes the datagram decrements this number by 1. If this value, after being decremented, is zero, the router discards the datagram. This field is needed because routing tables in the Internet can become corrupted. A datagram may travel between two or more routers for a long time without ever getting delivered to the destination host. This field limits the lifetime of a datagram. Another use of this field is to intentionally limit the journey of the packet. For example, if the source wants to confine the packet to the local network, it can store 1 in this field. When the packet arrives at the first router, this value is decremented to 0, and the datagram is discarded.

- Protocol. This 8-bit field defines the higher-level protocol(TCP, UDP etc) that uses the services of the IPv4 layer. Since the IPv4 protocol carries data from different other protocols, the value of this field helps the receiving network layer know to which protocol the data belong.

- Checksum. This is used to error detection. First, the value of the checksum field is set to 0. Then the entire header is divided into 16-bit sections and added together. The result (sum) is complemented and inserted into the checksum field.

- Source address. This 32-bit field defines the IPv4 address of the source. This field must remain unchanged during the time the IPv4 datagram travels from the source host to the destination host.

- Destination address. This 32-bit field defines the IPv4 address of the destination. This field must remain unchanged during the time the IPv4 datagram travels from the source host to the destination host.

- Options. Options, as the name implies, are not required for a datagram. They can be used for network testing and debugging. Although options are not a required part of the IPv4 header, option processing is required of the IPv4 software. This means that all implementations  must be able to handle options if they are present in the header.

Fragmentation

Maximum Transfer Unit (MTU)

Each data link layer protocol has its own frame format in most protocols. One of the fields defined in the format is the maximum size of the data field. The total size of the datagram must be less than this maximum size.The value of the MTU depends on the physical network protocol. For other physical networks, we must divide the datagram to make it possible to pass through these networks. This is called fragmentation.

When a datagram is fragmented, required parts of the header must be copied by all fragments. The option field mayor may not be copied, as we will see in the next section. The host or router that fragments a datagram must change the values of three fields: flags, fragmentation offset, and total length. The rest of the fields must be copied. Of course, the value of the checksum must be recalculated regardless of fragmentation.

Fields Related to Fragmentation

- Identification. This 16-bit field identifies a datagram originating from the source host. When a datagram is fragmented, the value in the identification field is copied to all fragments. In other words, all fragments have the same identification number, the same as the original datagram. The identification number helps the destination in reassembling the datagram. It knows that all fragments having the same identification value must be assembled into one datagram.

- Flags. This is a 3-bit field. The first bit is reserved. The second bit is called the do not fragment bit. If its value is 1, the machine must not fragment the datagram. If it cannot pass the datagram through any available physical network, it discards the datagram and sends an ICMP error message to the source host. If its value is 0, the datagram can be fragmented if necessary. The third bit is called the more fragment bit. If its value is 1, it means the datagram is not the last fragment; there are more fragments after this one. If its value is 0, it means this is the last or only fragments.

- Fragmentation offset. This 13-bit field shows the relative position of this fragment with respect to the whole datagram. It is the offset of the data in the original datagram measured in units of 8 bytes.

It is obvious that even if each fragment follows a different path and arrives out of order, the final destination host can reassemble the original datagram from the fragments received (if none of them is lost) by using the following strategy:

- The first fragment has an offset field value of zero.

- Divide the length of the first fragment by 8. The second fragment has an offset value equal to that result.

- Divide the total length of the first and second fragments by 8. The third fragment has an offset value equal to that result.

- Continue the process. The last fragment has a more bit value of 0.

IPv6 protocol

IPv4 has some deficiencies (listed below) that make it unsuitable for the fast-growing Internet.

- Address depletion is still a long-term problem in the Internet.

- The Internet must accommodate real-time audio and video transmission. This type of transmission requires minimum delay strategies and reservation of resources not provided in the IPv4 design.

- The Internet must accommodate encryption and authentication of data for some applications. No encryption or authentication is provided by IPv4.

To overcome these deficiencies, IPv6 (Internetworking Protocol, version 6), also known as IPng (Internetworking Protocol, next generation), was proposed. In IPv6, the Internet protocol was extensively modified to accommodate the unforeseen growth of the Internet. The format and the length of the IP address were changed along with the packet format.

Advantages

- Larger address space. An IPv6 address is 128 bits long but IPv4 uses only 32-bit address

- Better header format. IPv6 uses a new header format in which options are separated from the base header and inserted, when needed, between the base header and the upper-layer data. This simplifies and speeds up the routing process because most of the options do not need to be checked by routers.

- New options. IPv6 has new options to allow for additional functionalities.

- Allowance for extension. IPv6 is designed to allow the extension of the protocol if required by new technologies or applications.

- Support for resource allocation. In IPv6, the type-of-service field has been removed, but a mechanism (called flow label) has been added to enable the source to request special handling of the packet. This mechanism can be used to support traffic such as real-time audio and video.

- Support for more security. The encryption and authentication options in IPv6 provide confidentiality and integrity of the packet.

Packet Format

Each packet is composed of a mandatory base header followed by the payload. The payload consists of two parts: optional extension headers and data from an upper layer. The base header occupies 40 bytes, whereas the extension headers and data from the upper layer contain up to 65,535 bytes of information.

Base Header

- Version. This 4-bit field defines the version number of the IP. For IPv6, the value is 6.

- Priority. The 4-bit priority field defines the priority of the packet with respect to traffic congestion.

- Flow label. The flow label is a 3-byte (24-bit) field that is designed to provide special handling for a particular flow of data.

- Payload length. The 2-byte payload length field defines the length of the IP datagram excluding the base header.

- Next header. The next header is an 8-bit field defining the header that follows the base header in the datagram. The next header is either one of the optional extension headers used by IP or the header of an encapsulated packet such as UDP or TCP. Each extension header also contains this field. Note that this field in version 4 is called the protocol.

- Hop limit. This 8-bit hop limits field serves the same purpose as the TIL field in IPv4.

- Source address. The source address field is a 16-byte (128-bit) Internet address that identifies the original source of the datagram.

- Destination address. The destination address field is a 16-byte (128-bit) Internet address that usually identifies the final destination of the datagram. However, if source routing is used, this field contains the address of the next router.

Comparison between IPv4 and IPv6 Headers

- The header length field is eliminated in IPv6 because the length of the header is fixed in this version.

- The service type field is eliminated in IPv6. The priority and flow label fields together take over the function of the service type field.

- The total length field is eliminated in IPv6 and replaced by the payload length field.

- The identification, flag, and offset fields are eliminated from the base header in IPv6. They are included in the fragmentation extension header.

- The TTL field is called hop limit in IPv6.

- The protocol field is replaced by the next header field.

- The header checksum is eliminated because the checksum is provided by upper-layer protocols; it is therefore not needed at this level.

- The option fields in IPv4 are implemented as extension headers in IPv6.

Priority in IPv6

The priority field of the IPv6 packet defines the priority of each packet with respect to other packets from the same source. For example, if one of two consecutive datagrams must be discarded due to congestion, the datagram with the lower packet priority will be discarded. IPv6 divides traffic into two broad categories: congestion-controlled and noncongestion-controlled.

Congestion-Controlled Traffic If a source adapts itself to traffic slowdown when there is congestion, the traffic is referred to as congestion-controlled traffic. For example, TCP, which uses the sliding window protocol, can easily respond to traffic. In congestion-controlled traffic, it is understood that packets may arrive delayed, lost, or out of order. Congestion-controlled data are assigned priorities from 0 to 7. A priority of 0 is the lowest; a priority of 7 is the highest.

The priority descriptions are as follows:

- No specific traffic. A priority of 0 is assigned to a packet when the process does not define a priority.

- Background data. This group (priority 1) defines data that are usually delivered in the background. Delivery of the news is a good example.

- Unattended data traffic. If the user is not waiting (attending) for the data to be received, the packet will be given a priority of 2. E-mail belongs to this group. The recipient of an e-mail does not know when a message has arrived. In addition, an e-mail is usually stored before it is forwarded. A little bit of delay is of little consequence.

- Attended bulk data traffic. A protocol that transfers data while the user is waiting (attending) to receive the data (possibly with delay) is given a priority of 4. FTP and HTTP belong to this group.

- Interactive traffic. Protocols such as TELNET that need user interaction are assigned the second-highest priority (6) in this group.

- Control traffic. Control traffic is given the highest priority (7). Routing protocols such as OSPF and RIP and management protocols such as SNMP have this priority.

Noncongestion-Controlled Traffic This refers to a type of traffic that expects minimum delay. Discarding of packets is not desirable. Retransmission in most cases is impossible. In other words, the source does not adapt itself to congestion. Real-time audio and video are examples of this type of traffic.

Priority numbers from 8 to 15 are assigned to noncongestion-controlled traffic. Although there are not yet any particular standard assignments for this type of data, the priorities are usually based on how much the quality of received data is affected by the discarding of packets. Data containing less redundancy (such as low-fidelity audio or video) can be given a higher priority (15). Data containing more redundancy (such as high-fidelity audio or video) are given a lower priority (8).

Flow Label

A sequence of packets, sent from a particular source to a particular destination that needs special handling by routers is called a flow of packets. The combination of the source address and the value of the flow label uniquely define a flow of packets.

To a router, a flow is a sequence of packets that share the same characteristics, such as traveling the same path, using the same resources, having the same kind of security, and so on. A router that supports the handling of flow labels has a flow label table. The table has an entry for each active flow label; each entry defines the services required by the corresponding flow label. When the router receives a packet, it consults its flow label table to find the corresponding entry for the flow label value defined in the packet. It then provides the packet with the services mentioned in the entry. However, note that the flow label itself does not provide the information for the entries of the flow label table; the information is provided by other means such as the hop-by-hop options or other protocols.

In its simplest form, a flow label can be used to speed up the processing of a packet by a router. When a router receives a packet, instead of consulting the routing table and going through a routing algorithm to define the address of the next hop, it can easily look in a flow label table for the next hop.

In its more sophisticated form, a flow label can be used to support the transmission of real-time audio and video. Real-time audio or video, particularly in digital form, requires resources such as high bandwidth, large buffers, long processing time, and so on. A process can make a reservation for these resources beforehand to guarantee that real-time data will not be delayed due to a lack of resources. The use of real-time data and the reservation of these resources require other protocols such as Real-Time Protocol (RTP) and Resource Reservation Protocol (RSVP) in addition to IPv6.

To allow the effective use of flow labels, three rules have been defined:

- The flow label is assigned to a packet by the source host. The label is a random number between 1 and 224 - 1. A source must not reuse a flow label for a new flow while the existing flow is still active.

- If a host does not support the flow label, it sets this field to zero. If a router does not support the flow label, it simply ignores it.

- All packets belonging to the same flow have the same source, same destination, same priority, and same options

Extension Headers

The length of the base header is fixed at 40 bytes. However, to give greater functionality to the IP datagram, the base header can be followed by up to six extension headers. Many of these headers are options in IPv4. Six types of extension headers have been defined as follows

- Hop-by-Hop Option The hop-by-hop option is used when the source needs to pass information to all routers visited by the datagram. So far, only three options have been defined: Padl, PadN, and jumbo payload. The Padl option is 1 byte long and is designed for alignment purposes.  PadN is similar in concept to Pad1. The difference is that PadN is used when 2 or more bytes is needed for alignment. The jumbo payload option is used to define a payload longer than 65,535 bytes.

- Source Routing  The source routing extension header combines the concepts of the strict source route and the loose source route options of IPv4.

- Fragmentation The concept of fragmentation is the same as that in IPv4. However, the place where fragmentation occurs differs. In IPv4, the source or a router is required to fragment if the size of the datagram is larger than the MTU of the network over which the datagram travels. In IPv6, only the original source can fragment. A source must use a path MTU discovery technique to find the smallest MTU supported by any network on the path. The source then fragments using this knowledge.

- Authentication The authentication extension header has a dual purpose: it validates the message sender and ensures the integrity of data.

- The encrypted security payload (ESP) is an extension that provides confidentiality and guards against eavesdropping.

- Destination Option The destination option is used when the source needs to pass information to the destination only. Intermediate routers are not permitted access to this information.

Comparison between IPv4 Options and IPv6 Extension Headers

- The no-operation and end-of-option options in IPv4 are replaced by Padl and PadN options in IPv6.

- The record route option is not implemented in IPv6 because it was not used.

- The timestamp option is not implemented because it was not used.

- The source route option is called the source route extension header in IPv6.

- The fragmentation fields in the base header section of IPv4 have moved to the fragmentation extension header in IPv6.

- The authentication extension header is new in IPv6.

- The encrypted security payload extension header is new in IPv6.

TRANSITION FROM IPv4 TO IPv6

Because of the huge number of systems on the Internet, the transition from IPv4 to IPv6 cannot happen suddenly. It takes a considerable amount of time before every system in the Internet can move from IPv4 to IPv6. The transition must be smooth to prevent any problems between IPv4 and IPv6 systems Three strategies have been devised to help the transition .

- Dual Stack

It is recommended that all hosts, before migrating completely to version 6, have a dual stack of protocols. In other words, a station must run IPv4 and IPv6 simultaneously until all the Internet uses IPv6. To determine which version to use when sending a packet to a destination, the source host queries the DNS. If the DNS returns an IPv4 address, the source host sends an IPv4 packet. If the DNS returns an IPv6 address, the source host sends an IPv6 packet.

- Tunneling

Tunneling is a strategy used when two computers using IPv6 want to communicate with each other and the packet must pass through a region that uses IPv4. To pass through this region, the packet must have an IPv4 address. So the IPv6 packet is encapsulated in an IPv4 packet when it enters the region, and it leaves its capsule when it exits the region. It seems as if the IPv6 packet goes through a tunnel at one end and emerges at the other end. To make it clear that the IPv4 packet is carrying an IPv6 packet as data, the protocol value is set to 41.

- Header Translation

Header translation is necessary when the majority ofthe Internet has moved to IPv6 but some systems still use IPv4. The sender wants to use IPv6, but the receiver does not understand IPv6. Tunneling does not work in this situation because the packet must be in the IPv4 format to be understood by the receiver. In this case, the header format must be totally changed through header translation. The header of the IPv6 packet is converted to an IPv4 header

Header Translation Procedure

1. The IPv6 mapped address is changed to an IPv4 address by extracting the rightmost 32 bits.

2. The value of the IPv6 priority field is discarded.

3. The type of service field in IPv4 is set to zero.

4. The checksum for IPv4 is calculated and inserted in the corresponding field.

5. The IPv6 flow label is ignored.

6. Compatible extension headers are converted to options and inserted in the IPv4 header. Some may have to be dropped.

7. The length of IPv4 header is calculated and inserted into the corresponding field(HLEN)

8. The total length of the IPv4 packet is calculated and inserted in the corresponding field